This document contains lsi53c8xx-specific documentation information.

Please see also :

- ReleaseNotes/lsi53c8xx_dev_relnotes

How to use

==========

- To use the driver, just drop an LSI/NCR/SYM53Cxxx SCSI board

into any PCI slot, connect it to SCSI drives, make sure

the termination chain is set properly, and add the driver

to your kicklayout.

If UBoot can see your drives then the driver should too.

- This driver can work happily aside the other OS4 disk

drivers.

The driver supports up to 4 PCI chips / boards.

Thanks to Max (soft3) for the SCSI board, and to Ferran Garcia

for his invaluable testing efforts.

This driver has been successfully tested by Ferran with :

- Iomega ZIP100 drive (Ferran)

- Epson GT7000 scanner with BetaScan 2 and 3 (ignore the DSI's in the Epson driver) ;

ScanQuix 3 and 5, including any program using ARexx hooks or other means to load

through it : ADPro, fxScan v2 ImageFX v4.5 ;

ImageFX v4.5 using its own driver

This driver has been successfully tested by myself with :

- IBM 4.3 GB UWSCSI harddrive holding FFS & SFS partitions from a Classic system

- Epson GT7000 scanner with BetaScan 3 (ignore the DSI's in the Epson driver)

- Wangtek DC6525 tape drive with Diavolo Backup (auto detection)

- Wangtek DC6120 tape drive with Diavolo Backup (auto detection)

- Wangtek DC6150 tape drive with Diavolo Backup (auto detection)

- Fujitsu H2249 330 MB ESDI harddrive and EMULEX ESDI to SCSI attachment

board from an old VAX

- IBM CDRM001 CD-Rom reader

- TEAC FC-1 SCSI floppy drive

- Apple SC-300 CD-Rom drive

- NeXT black external CD-Rom drive

- various other harddrives from a Silicon Graphics Personal Iris

I - Configuring lsi53c8xx through UBOOT envvars

================================================

Currently lsi53c8xx is configurable through the following UBOOT envvars :

- scsi_pci_override

Legal values are : set to the PCI product ID if your board does not show

a product id of 000F. For example, my LSI53C895 board has a product id

of 000C thus I used that feature.

Note : Uboot's internal SCSI code uses the same mechanism and the same

envvar (this is why I chose that name : if you have properly set it for

UBoot then it works for the OS4 driver).

Default if unset : 000F.

- lsi53c8xx_verbose

Legal values are : set to anything makes the driver verbose at boot time.

Default if unset : driver silent.

- lsi53c8xx_timeout

Legal values are between 1 and 30 (seconds).

Default : if you don't set this envvar, lsi53c8xx will use a 20 second timeout.

Don't forget the 'saveenv', as lsi53c8xx uses nonvolatile.library,

which only accesses the envvars stored in nvram.

The driver supports up to 4 lsi53c8xx PCI boards / chips. For the 2nd, 3rd etc.

chips, add '.2', '.3' etc. to envvar names. I.e., setenv lsi53c8xx_conf.2 12

II - Notes

==========

- This configurability feature relies on two kickstart modules from Ross

Vumbaca that MUST be listed in your kicklayout : nonvolatile.library.kmod

50.2+ and nvram.resource.kmod. If those modules are not present, lsi53c8xx will

revert to default values.

- These envvars HAVE to be saved ('saveenv' after modifying them), because

they are read directly from the nvram.

- To delete an envvar, just define a blank content & then saveenv (ie

'setenv lsi53c8xx_conf' and then 'saveenv' will delete this envvar).

- When you grab a boot log & send it to me, please use a debuglevel of 1.

III - Fine Art of drive partitioning

===================================

A few things you must know :

- You'll get decent performance from any FS only with buffers >= 500

- You'll get decent performance from FFS only with blocksize >= 2048

- You'll get the best performance from SFS only with blocksize = 512

- SFS is way faster than FFS in its default (non-caching) behaviour,

but FFS is said to be safer.

- SFS + its diskcache.library is even faster.

- There is a cache plugin for FFS, and you can also set FFS in

writeback cache mode. Using both things make it waaaaay faster

(almost comparable to SFS). The FFS cache plugin is 'fs_cache_plugin',

you will find it somewhere on your OS4 CD, just 'run fs_cache_plugin DH0:'

for each FFS partition you wish to run with cache.

- The command I use to bench r/w perf is scsispeed, found in the

latest diskspeed archive from Aminet, and also in an OS4 native

form on the OS4 beta server (quick port by myself, dropped the

m68k CPU usage measurement code).

Make sure to create your SFS partition this way :

- 500 buffers

- blocksize set to 512

- maxtransfer set to 7FFFFFFF

- mask set to FFFFFFFE