PROJECT:	T1P1 Activity on Lawful Intercept for UMTS

TITLE:	Recommended changes to TS 33.108 to address Issue #3, Roaming, of the issues list documented in T1P1/2002-004.

SOURCE:	Lou Degni

CALEA Implementation Section

14800 Conference Center Drive, Suite 300

Chantilly, VA 20151-3810

Tel:  	(703) 814-4729

Fax:  	(703) 814-4720

Sandra Lopez

Telcordia Technologies, Inc.

331 Newman Springs Road

Red Bank, NJ 07701

Tel:	(732) 758-3222

Fax:  	(732) 758-4196

e-mail:	slopez@telcordia.com

DATE:	January 23, 2002

LOCATION: 	Kiawah Island, South Carolina

ABSTRACT:	Proposed text to address Issue #3, Roaming, of the TS 33.108 issues list documented in T1P1/2002-004.

DISTRIBUTION:	T1P1, T1P1.SAH

NOTICE

©2002 Telcordia Technologies, Inc. The proposals in this submission have been formulated to assist committee T1P1.  This document is offered to the committee as a basis for discussion and is not binding on Telcordia Technologies. The requirements are subject to change in form and numerical value after more study. Telcordia Technologies specifically reserves the right to add to, or amend, the quantitative statements made herein.  Nothing contained herein shall be construed as conferring by implication, estoppel, or otherwise any license or right under any patent, whether or not the use of information herein necessarily employs an invention of any existing or later issued patent.

[PAGE INTENTIONALLY LEFT BLANK]

A. Introduction

This contribution proposes text to address shortcomings in TS 33.108 in area of reporting roaming related events. This contribution addresses Issue # 3 of the issues list documented in T1P1/2002-004 and action items assigned to CIS/Telcordia during the T1P1.SAH conference calls held on January 8, 2002 and January 14, 2002.

B. Discussion

Issue:  TS 33.108 does not support the capability to report the new “Serving System” when the intercept subject roams. In addition, TS 33.108 does not explicitly describe that it will continue to deliver content of communication to the law enforcement monitoring facility (LEMF) when the content of communication is still available at the home network.

Discussion: GPRS networks can support two basic roaming scenarios:

- Mobile Station (MS) connects via the Visiting Serving GPRS Support Node (VSGSN) and Home Gateway GPRS Support Node (HGGSN);

- MS connects via VSGSN and Visiting GGSN (VGGSN).

Precedent is established in J-STD-025 to report roaming events and continue delivering content of communication intercept, when appropriate and available (e.g., SMS cases).

Requirement: This capability is required by the CALEA Law, which has the following provision for this capability:

“A telecommunications carrier offering a feature or service that allows subscribers to redirect, hand off, or assign their wire or electronic communications to another service area or another service provider or to utilize facilities in another service area or of another service provider shall ensure that, when the carrier that had been providing assistance for the interception of wire or electronic communications or access to call-identifying information pursuant to a court order or lawful authorization no longer has access to the content of such communications or call-identifying information within the service area in which interception has been occurring as a result of the subscriber's use of such a feature or service, information is made available to the government (before, during, or immediately after the transfer of such communications) identifying the provider of wire or electronic communication service that has acquired access to the communications.”

Law enforcement agencies need access to all available authorized content of communications and IRI, and any additional information that would assist law enforcement in determining the service area or other carrier(s) that have access to any information. Law enforcement agencies (LEAs) need:

- access to the intercept subject’s communications until the carrier’s network no longer has access to the communication.

- Network Operators (NOWs)/ Access Provider (APs)/ Service Providers (SvPs) to provide as part of Intercept Related Information (IRI) the identity of the new NOW/AP/SvP and/or service area to law enforcement when access to an intercept subject’s communications cannot be maintained. The identity of the new NOW/AP/SvP and/or service area should be provided to law enforcement as soon as it is available.

- to be provided with any information that will permit the LEA to determine or infer the new NOW/AP/SvP’s or service area’s identity if this information is unavailable.

Recommendation: The author recommends the addition of new triggers to report the roaming event and related information.  Changes must be made o TS 33.107 and TS 33.108.

- Proposed Changes to TS 33.107 to address Roaming

  - Changes to Clause 7.2.1 of TS 33.107

Add the following as the new last paragraph for this Clause:

In the case where the 3G GGSN is performing interception of the content of communications, the interception subject roams, and the 3G GGSN continues to be involved in the handling of the content of communications, the 3G GGSN shall continue to perform the interception of the content of communications.

  - Changes to Clause 7.3 of TS 33.107

Change the first two paragraphs as follows:

Intercept Related Information (Events) are necessary at the Mobile Station Attach, Mobile Station Detach, PDP Context Activation, Start of intercept with PDP context active, PDP Context Deactivation, RA update, Roaming including Home 3G GGSN, and SMS events.

Figure 21 shows the transfer of intercept related information to the DF2. If an event for / from a mobile subscriber occurs, the 3G GSN or the Home Location Register (HLR) sends the relevant data to the DF2.

Also, the diagram for FIGURE 21 must be modified to include the (HLR) as a source of IRI.

Figure 21: Provision of Intercept Related Information

  - Changes to Clause 7.3.1 of TS 33.107

The following information needs to be transferred from the 3G GSN or the 3G HLR to the DF2 in order to allow a DF2 to perform its functionality:

  - Changes to Clause 7.3.2 of TS 33.107

Change the first paragraph as follows:

There are seven eight different events in which the information is sent to the DF2 if this is required. Details are described in the following section. The events for interception are configurable (if they are sent to DF2) in the 3G GSN or the HLR and can be suppressed in the DF2.

Add the following text:

The following events are applicable to the HLR:

- Roaming.

Change the paragraph immediately preceding Table 2 as follows:

A set of fields as shown below is used to generate the events. The events transmit the information from 3G GSN or HLR to DF2. This set of fields as shown below can be extended in the 3G GSN or HLR, if this is necessary as a national option. DF2 can extend this information if this is necessary as a national option e.g. a unique number for each surveillance warrant.

  - Changes to Table 2 of TS 33.107

Event Type Description which type of event is delivered: MS attach, MS detach, PDP context activation, Start of intercept with PDP context active, PDP context deactivation, SMS, Cell and/or RA update, Roaming.

Event Date Date of the event generation in the 3G GSN or the HLR.

Event Time Time of the event generation in the 3G GSN or the HLR.

Old Network Identifier Identity of the old PLMN servicing the intercept subject.

New Network Identifier Identity of the new PLMN servicing the intercept subject.

New Network Address New network address providing service to the intercept subject.

  - Addition of Clause 7.4.10 to TS 33.107

- 7.4.10	Roaming

The Roaming report event is generated at the HLR, when the HLR has detected that the intercept subject has roamed.  The fields will be delivered to the DF2 if available:

Observed MSISDN

Observed IMSI

Observed IMEI

PDP address of observed party

Event Type

Event Time

Event Date

Correlation number

Access Point Name

PDP Type

Old Network Identifier

New Network identifier

New Network Address

IAs (if applicable)

- Proposed Changes to TS 33.108 to address Roaming

  - Changes to Clause 6.5 of TS 33.108

The following text changes need to be incorporated into Clause 6.5 of TS 33.108:

There are eight nine different event types received at DF2 level. According to each event, a Record is sent to the LEMF if this is required. The following table gives the mapping between event type received at DF2 level and record type sent to the LEMF.

- Table 1: Mapping between UMTS Data Events and HI2 records type

| Event | IRI Record Type |
| --- | --- |
| GPRS attach | REPORT |
| GPRS detach | REPORT |
| PDP context activation (successful) | BEGIN |
| PDP context activation (unsuccessful) | REPORT |
| Start of intercept with PDP context active | BEGIN |
| PDP context deactivation | END |
| Cell and /or RA update | REPORT if no PDP context is active CONTINUE if, at least, one PDP context is active |
| SMS | REPORT |
| Roaming | REPORT |

  - Additions to Table 2 of TS 33.108

The following additional parameters need to be added to Table 2 of TS 33.108:

- Table 2: Mapping between Events information and IRI information

| parameter | description | HI2 ASN.1 parameter |
| --- | --- | --- |
| Old Network Identifier | Reports the old NOW/AP/SvP and old network element identifier. | networkIdentifier |
| New Network Identifier | Reports the new NOW/AP/SvP and new network element identifier. | networkIdentifier |
| New Network Address | This field provides the service area identity, RAI and/or location area identity known to the HLR at the time of event record production. | locationOfTheTarget |

  - Changes to Clause 6.5.1.1: REPORT record information

Add a new IRI triggering event for the REPRT record to be issued to report information about the NWO/AP/SvP providing service to the interception subject with terminal mobility, when the terminal is authorized for service with another NWO/AP/SvP or in another service area. A new bullet needs to be added as shown below:

- a mobile station is authorized for service with another NWO/AP/SvP or in another service area.  The REPORT record shall be generated by the HLR.

Table 6-8: Roaming REPORT Record

| Parameter | MOC | Description/Conditions |
| --- | --- | --- |
| observed MSISDN | C | Provide at least one and others when available. |
| observed IMSI |  |  |
| observed IMEI |  |  |
| observed PDP address | C | Provide to identify the PDP address of the intercept subject. |
| event type | C | Provide Roaming event type. |
| event date | M | Provide the date and time the event is detected. |
| event time |  |  |
| PDP type | C | Provide to describe the PDP type of the observed PDP address. |
| old network identifier | M | Shall be provided. |
| new network identifier | M | Shall be provided. |
| lawful intercept identifier | M | Shall be provided. |
| new network address | M | Provides the service area identity, RAI and/or location area identity known to the HLR at the time of event record production. |

  - Add a Clause to address provision of IRI at 3G GGSN when roaming

Add Clause 6.6 to address reporting IRI when intercepting at the 3G GGSN for roaming scenarios.

## 6.6	IRI for packet domain at 3G GGSN

When the interception subject is authorized for service with another NWO/AP/SvP or in another service area and interception is occurring at the Home GGSN, the Home GGSN is capable of continuing to perform limited surveillance on the interception subject ongoing communications.

The Home GGSN shall report IRI for the following events when the interception subject is authorized for service with another NWO/AP/SvP or in another service area and the interception subject’s communications continue to be routed through the Home GGSN:

-	PDP context activation;

-	PDP context deactivation;

-	Start of interception with PDP context active;

-	Packet Activity.

  - Add a Clause to address Provision of Content of Communication When 3G GGSN Interception is active and Intercept Subject is Roaming

Add Clause 6.7 to address Content of Communication Interception at the 3G GGSN for roaming scenarios.

## 6.7	Content of Communication for packet domain

When the interception subject is authorized for service with another NWO/AP/SvP or in another service area and interception is occurring at the Home GGSN, the Home GGSN is capable of continuing to perform surveillance on the interception subject ongoing communications.

The Home GGSN shall deliver content of communication (if LEA is authorized to receive such information) when the interception subject is authorized for service with another NWO/AP/SvP or in another service area and the interception subject’s communications continue to be routed through the Home GGSN.

  - Additions to Clause B.4 of TS 33.108

The ASN.1 encoding in Clause B.4 of TS 33.108 must be modified to define the new parameters required to be reported with the Roaming event.

oldNetworkIdentifier		[31?] Network-Identifier OPTIONAL,

newNetworkIdentifier		[32?] Network-Identifier OPTIONAL,

newNetworkAddress				[33?] Location OPTIONAL,

--location of the target subscriber known to the HLR when

--Roaming event is detected.

The ASN.1 encoding of the GPRSEvent parameter in TS 33.108 needs to be modified to add a codepoint for this event (See Clause B.2.6). Within the ASN.1, the following value should be added

GPRSEvent ::= ENUMERATED

{

pDPContextActivation 					(1),

startOfInterceptionWithPDPContextActive	(2),

pDPContextDeactivation					(4),

gPRSAttach 								(5),

gPRSDetach 								(6),

locationInfoUpdate 						(10),

sMS 									(11),

roaming									(13?),

...

}

-- see ref [10]

C. Recommendation

Approve all recommended changes.

Roaming is defined as the ability for a cellular customer to automatically make & receive voice calls, send & receive data, or access other services when traveling outside the geographic coverage area of the home network, by means of using a visited network. Roaming is technically supported by mobility management, authentication and billing procedures. Establishing roaming between network operators is based on - and the commercial terms are contained in - Roaming Agreements.