%N obstau

%D Computes tau and sky temperature from atmospheric model

%P mchw

%: utility

%B

OBSTAU computes tau, sky temperature and single sideband

systemp from an atmospheric model.

The program assumes scale height for water is 1.6 km.

You must specify the water content of the atmosphere.

This can be input as precipitable water (mm),

or relative humidity (%) and temperature (C).

Output: zenith opacity, estimated skytemp at current airmass,

and single sideband system temperature, Tsys.

%A altitude

Altitude of observatory (km). Default=1 km.

%A freq

freq (GHz). Default=100 GHz.

%A mmh2o

precipitable water (mm). Default=10 mm.

%A relhumid

ground level relative humidity (%). Default=0. Uses mmh2o.

A non-zero relhumid is used to calcuate mmh2o.

%A airtemp

ground level temperature (C). Default=0.

%A airmass

airmass in units of zenith airmass. Default=1 airmass.

%A elev

elevation in degrees used to compute airmass if elev.ne.0.

Default elev=0, i.e. use given airmass.

%A trx

Double sideband receiver noise temperature. Default=40 K.

or if (freq.gt.120) trx=80 K is default.

This is used to calculate the single sideband systemp as:

systemp = 2.* (skytemp+trx) * exp(airmass*tauzenith)