%N uvplanet

%D Fiddle planetary and solar system data.

%P rjs

%: uv analysis

%B

uvplanet is a Miriad task used to fiddle visibility datasets

from planetary (and other solar system) observations. It can:

- Add planet orientation parameters (plmaj,plmin,plangle) to

the dataset. uvplanet recognises planets by their source name,

and knows their orientations as a function of time.

- Apply a time-varying phase shift to track the object.

- Subtract a thermal disk.

- Subtract background sources, accounting for a moving phase centre.

%A vis

The name of the input uv datasets. Several can be given

Wildcards are supported. No default.

%A out

The name of the output uv data-set. There is no default name.

%A select

The normal uv selection commands. The default is to select everything.

%A pltb

Parameters for planets only, which determine model for the planetary

disk to be subtracted off. It consists of one or two values. The

first is the blackbody temperature of the disk, in Kelvin.

The second is the limb darkening parameter. The limb darkening

parameter is a value between 0 and 1. The apparent brightness of

the disk falls off as cos(theta)**limb.

uvplanet has built-in ephemerides and models of the planets which

gives their apparent sizes and shapes.

The default is 0 for both parameters (i.e. do not subtract off a disk).

%A options

This gives extra processing options. Several options can be given,

each separated by commas. They may be abbreviated to the minimum

needed to avoid ambiguity. Possible options are:

replace     Replace the data with the blackbody disk and point

sources (the normal behaviour is to subtract the

blackbody disk and point sources).

pparam      Normally uvplanet uses its own ephemerides to determine

planet major and minor axes and position angle to model

the planets disk. The pparam option causes uvplanet to use

magnetic    For Jupiter only: The magnetic axis is used to

set planet orientation parameters. This overrides the

normal behaviour of using the spin axis.

arcane      The format for the pmotion text files is the

"arcane" format (see below).

The following options can be used to turn off calibration corrections.

The default is to apply any calibration present.

nocal       Do not apply the gains table.

nopass      Do not apply bandpass corrections.

nopol       Do not apply polarization corrections.

%A sources

This gives the position and flux of point sources to be subtracted

from the visibility dataset. Many sources can be given. Each

source is specified by three numbers:

ra dec flux

The RA and DEC are in normal Miriad format, and the flux is in Jy.

%A pmotion

This parameter allows phase shifts to be applied to the data to

make the phase centre track the object of interest. Assuming that

the object has significant proper motion, this can

correct for phase tracking errors (e.g. wrong ephemeris) or perhaps

no tracking at all. One or two tracking files can be given. The first

file gives the centre of the object (as a function of time). The second

gives the phase centre of the observation (as a function of time).

If both files are given, the data are phase shifted to the centre

of the object (i.e. corrects for tracking errors). If only one file

is given, then the RA and DEC in the dataset are taken as the

phase centre of the telescope.

The "pmotion" parameter can also be used to correct the RA and DEC

stored in the dataset. If these are incorrect (e.g. many telescopes

and the FITS format, do not store the time-varying RA/DEC, even though

they track time varying RA/DEC), then giving the one text file

twice will cause the RA and DEC to be corrected and no shifts being

applied.

The text files contain potentially many lines, with each line

giving a sky position at a given time.

Two formats for the text files are possible (if there are

two input files, both must be in the same format). The normal format

consists of 3 values per line, being

time ra dec

with "time" (UTC), "ra" and "dec" being the position of the object at

a given time. The time, ra and dec are in the normal Miriad formats.

The times should be in increasing order. Linear interpolation

is used to estimate positions between the given times.

A convenient way to develop this file is through the JPL ``Horizons''

service. This is accessed through

telnet ssd.jpl.nasa.gov 6775

Using a text reformating tool (e.g. PERL), the output from Horizons

is readily converted to the format required by uvplanet.

Documentation on the Horizons system is available from

http://ssd.jpl.nasa.gov/horizons.html

The other possible format is the ``arcane'' format, which will be

expected if "options=arcane" is given. This format is provided for

obscure historical support. It consists of 6 values per line, being

start_time time ra dec dra ddec

The times are in modifed Julian days, ra and dec are in turns,

and the rate parameters, "dra" and "ddec", are in turns/day. The

record is considered valid after "start_time", and the rates

are used to estimate positions between given times.