%N smaflux

%D Set the flux scale of a vis dataset given a planet obs.

%P jhz

%: calibration

%B

SmaFlux is a MIRIAD program which corrects the flux density scale

in visibility datasets. In doing this, it assumes that the flux

density scale is out by a constant factor. SmaFlux looks for

observations of planets, and given its model of the planetary

visibility function, it computes the factor needed to correct

the flux density scale of the data set. The values of the Planck

whole disk brightness temperature from the SMA planetary model

(http://sma1.sma.hawaii.edu/planetvis.html) are adopted for

brightness temperature of a planet or a moon which is included

in the following list:

Mercury, Venus, Mars, Jupiter, Saturn, Uranus, Neptune, Pluto,

Ganymede, Callisto, Titan, Ceres, Pallas, Vesta.

To fix the flux density scale, SmaFlux creates or modifies calibration

tables attached to each dataset.

To run this program, the data directory for the SMA planetary

model needs to be installed by excuting the following script:

cd $MIR/install

get_smaplmodel

%A vis

Input visibility datasets. Several datasets can be given (wildcards

are supported). The datasets should include observations of a planet.

%A select

Normal uv-selection parameter. This selects the data in the input

datasets to analyse. See the help on ``select'' for more information.

smaflux will use any data that has a source name which it recognises

as a planet. You may wish to select just the shortest spacing, where

the planet is strongest.

%A mirhome

location of MIRIAD's home directory;

The default is $MIR.

%A options

Extra processing options. Several can be given, separated by commas.

vector  Use the real part (rather than amplitude) of the data and

model. This option should be used if the visibility data

are phase calibrated.

noapply Do not apply the scale factor (just evaluate it).

nofqav  When dealing with amplitude data, SMAFLUX normally

averaging in frequency first, to avoid noise

biases. The nofqav disables this averaging.