%N passfit

%D Fit polynomials to a passband calibration set

%P lgm

%: calibration

%B

PASSFIT is a MIRIAD task that fits polynomials to the passband

calibration data.

The data and fits are displayed as amplitudes and phases with

all spectral windows for a baseline displayed at one time.

The amplitudes appear as red dots (open dots for flagged data);

the phases appear as yellow pluses (open triangles for flagged data).

The amplitudes are autoscaled; the phases range from -180 to 180

degrees from bottom-to-top of screen. Bad data may to flagged by deleting

the corresponding amplitude point with the cursor. If you do edit data

on a baseline, the passband is automatically refit and shown again when

you attempt to exit that baseline. The fits are shown as solid colored lines.

Commands in cursor mode are:

?   This help (also redraws screen)

q   quit (immediate quit without save)

e   exit (go to next baseline or exit with save)

d   delete point nearest cursor (amplitude ONLY)

a   add back point nearest cursor

1   numbers 1,2,3... zooms to that window

x   Toggle zoom around in x

y   Toggle zoom around in y

z   Toggle zoom around in x and y

u   unzooms in both x and y

%A pcal

The passband calibrator visibility dataset, produced by passmake.

No default.

%A drop

Number of channels to drop at the start and end of each window

before fitting. Fits will be extrapolated to cover these channels.

A different number of dropped channels for each window can be

given, up to a maximum of ``nspect/2'' (nspect is the total number of

spectral windows - see UVLIST - we assume USB and LSB). For negative

numbers it's absolute value will be the fraction of the window size

to be dropped at each side.

Default: -8 (1/8th of window dropped from each side)

Note: For mode 4, and additional 1/8th is dropped from the center.

%A order

The polynomial order for the amp and phase fit to each passband window.

Default: 1,1 (1st order for amplitude and 1st order for phase)

%A chave

Number of channels to average together before fitting polynomials

to amp and phase.  This is useful for poor signal-noise data to

reduce amplitude bias.  Data for individual channels are still displayed

in the plot; consequently, there may be an offset between the fits and

the points in the amplitude plots (amplitude bias).

Default: 1

%A split

A value of true results in mode 4 windows being split into 2 sub-windows

which are independently fit. False results in the standard fit of

one polynomial to each window. This is probably only useful for data

taken in 1993/94 with BIMA's old correllator.

Default: false.

%A device

This keyword is used to specify the plotting device. It uses

the normal PGPLOT format of

file/type

or

device/type

Usually there exists a default file name or device name.

The part after the slash gives the plotter type. Examples of the

type are:

xs    X-windows output. This window is resizeable and

persistent. Prepending a number allows you to use multiple

windows, e.g. 1/xs, 2/xs etc.

xd    X-windows output. A transient window that disappears

when your task exists.

ps    Postscript (landscape mode).

vps   Postscript (portrait mode).

cps   Color postscript (landscape mode).

gif   gif files. Only supported with g77, not gfortran.

vcps  Color postscript (portrait mode).

tek   Tektronix 4010 or equivalent.

xterm Xterm window (better for simple plots over slow X links).

A `?' as device name generates a complete list of the recognized

types (which will most likely vary from system to system).

Files created for hardcopy devices must be spooled to the printer

separately. The command needed for this is site and device specific.