%N elevcor

%D Correct ATCA data for elevation-dependent gain.

%P dpr

%: uv analysis

%B

ElevCor corrects ATCA data for elevation-dependent gain, using a

pre-defined template. This may be useful when the target and

secondary calibrator are sufficiently distant on the sky that a

gain elevation dependence cannot be subsumed into the usual time

dependent gains.

ElevCor scales the visibilities themselves, it doesn't mess with

the gains. Actually, elevcor doesn't copy or apply existing

gain/pol/bandpass tables at all. If you want to retain

pre-existing calibration (eg. bandpass), copy them after with

gpcopy. Of course, existing amplitude gains won't be very

useful.

Currently, the only templates available are for 3cm and 3mm data.

For details of the gain elevation-dependence template, consult

Hayley Bignall.

%A vis

The names of the input uv data set. No default.

%A out

The name of the output uv data set. No default.

%A options

Extra processing options, minimum match allowed:

simulate  Include a gain error to the data, rather

than applying the correction. This can be

used on data generated with uvgen to simulate

elevation-dependence in fake uvdata.

replace   Replace the data with the gain function.