%N uvflux

%D Determine some statistics about visibilities.

%P rjs

%: uv-analysis

%B

UvFlux is a MIRIAD task which gives a mean visibility value.

The visibilities are averaged together regardless of time, (u,v)

coordinate, frequency or baseline. Distinct polarisations

and sources, however, are averaged separately. This task will be

most useful when examining the fluxes of point sources.

Both vector and scalar averages are formed. It also prints out

the rms scatter around the vector mean, and the RMS variation

in the amplitude.

Definitions in Detail: UvFlux prints out six quantities.

Theoretical RMS -- This is the rms error, resulting from

receiver thermal noise, that would be expected in the real or

imaginary part of each visibility.

Vector Average -- This is simply the normal average (mean)

of the real and imaginary parts of the visibility data. For a

point source, the real part should give the point source flux

density, and the imaginary part should be noise.

RMS Scatter -- This is the RMS scatter of real and imaginary

parts of the actual visibilities around their mean value. If the

data do represent a point source, and it is well calibrated, this

should be the same number as the ``Theoretical RMS''.

Average Amplitude -- This gives the mean value of the visibilty

amplitude (the so-call amplitude scalar average). If the S/N

ratio is much less than 1, this average amplitude will be dominated

by thermal noise bias, and should be 1.25 times larger than the

``Theoretical RMS''. If the S/N ratio is much greater than 1, then

this should be a good estimate of the point-source's flux density.

This will be unaffected by phase calibration errors (unlike the

``Vector Average'', which decorrelates).

RMS Amp Scatter -- This gives the RMS scsatter of the visibility

amplitudes around the mean visibility amplitude. If the S/N ratio

is much less than 1, then this should be 0.65 times the ``Theoretical

RMS'. If the S/N ratio is much greater than 1, this should be the

same as ``Theoretical RMS''.

Number Corrs -- This gives the number of correlations, N, used in

forming each mean. NOTE that the three RMS values printed are RMS

difference between the correlations and some mean -- it is not

an error-of-the-mean. To convert an RMS to an error-in-the-mean,

divide by sqrt(N).

%A vis

The name of the input visibility data-set. Several files can be

given, wildcarding is supported. No default.

%A select

Standard uv selection. The default is all data.

%A line

Standard line-type specification. When there are multiple channels

selected, uvflux averages them all together.

%A stokes

Normal Stokes processing. You can select several Stokes or

polarisation parameters, which will be be averaged independently.

%A offset

An offset (arcsec) to shift the uv data. The sign convention is the

same as INVERT, MAXFIT, UVFIT, etc. The default is 0,0 (no shift).

%A options

Extra processing options. Several can be given, separated by

commas. Minimum match is used. Possible values are:

nocal    Do not apply any antenna gain calibration corrections.

By default these are applied if they exist.

nopol    Do not apply polarisation leakage corrections. By default

these are applied if they exist.

nopass   Do not apply bandpass corrections. By default these

are applied if they exist.

uvpol    Print out fractional linear polarisation and

polarisation position angle (provided Stokes I,Q,U are

requested).

vlbi     Average amplitude of vector average for each integration.

data are vector averaged over channel and baseline,

and amplitude averaged for each integration interval.