%N invertf

%D Transform multi-pointing visibility data into a map

%P wsrt

%: map making

%B

INVERTF is a MIRIAD task which forms images from visibilities.

INVERTF can form continuum images or spectral line cubes. It

can generate images/cubes for several polarisations, as well

as handling multi-frequency synthesis and mosaicing observations.

INVERTF can also form complex-valued images from non-Hermitian data

(e.g. holography data). Appropriate point-spread functions can also

be generated.

INVERTF is derived from INVERT by the WSRT group to experiment with

faster algorithms using no scratch space but more memory.

%A vis

Input visibility data files. Several files can be given. No default.

%A map

Output map (image) file name. Each output file consists of a single

polarization/Stokes parameter. If several different pols/Stokes

images are being made, then several file names should be given. No

default.

%A beam

Output beam (point-spread function) file name. The default is not

to make a beam.

%A imsize

The size of the output dataset. The default is to image out to

primary beam half power points. For options=mosaic, an image of

this size is made for each pointing before a linear mosaic

operation is performed.

%A cell

Image cell size, in arcsec. If two values are given, they give

the RA and DEC cell sizes. If only one value is given, the cells

are made square. The default is about one third of the resolution

of the resultant images.

%A offset

When not mosaicing, this gives the sky position to shift to the

center of the output images. The position is specified as an

offset (in arcsec) from the observing center. The default is to

perform no shifting.

When mosaicing, this gives the sky coordinate (RA and DEC) of the

reference pixel in the imaging process. The value can be given in the

form hh:mm:ss,dd:mm:ss, or as decimal hours and degrees. INVERT

applies appropriate shifts to make this location fall on a pixel.

The default is a central observing center.

%A fwhm

This determines a gaussian taper to apply to the visibility data.

It specifies the FWHM of an image-domain gaussian -- tapering the

visibility data is equivalent to convolving with this image-domain

gaussian.

Either one or two values can be given, in arcsec, being the FWHM in

the RA and DEC directions. If only one value is given, the taper is

assumed to be symmetric. The default is no taper.

The signal-to-noise ratio will be optimised in the output image if

this parameter is set to the FWHM of typical image features of

interest.

If you are more accustomed to giving this parameter in the uv plane

(as AIPS requires), then:

fwhm(image plane) = 182 / fwhm(uv plane)

where the image plane fwhm is measured in arcseconds, and the uv plane

fwhm is measured in kilowavelengths.

%A sup

Sidelobe suppression area, given in arcseconds. This parameter

gives the area around a source where INVERT attempts to suppress

sidelobes. Two values (for the RA and DEC directions respectively)

can be given. If only one value is given, the suppression area is

made square. The default is to suppress sidelobes in an area as

large as the field being mapped.

The suppression area is essentially an alternate way of specifying

the weighting scheme being used. Suppressing sidelobes in the entire

field corresponds to uniform weighting (so the default corresponds to

uniform weighting). Natural weighting gives the best signal to noise

ratio, at the expense of no sidelobe suppression. Natural weighting

corresponds to SUP=0. Values between these extremes give a tradeoff

between signal to noise and sidelobe suppression, and roughly

correspond to AIPS ``super-uniform'' weighting.

%A robust

Brigg's visibility weighting robustness parameter. This parameter

can be used to down-weight excessive weight being given to

visibilities in relatively sparsely filled regions of the $u-v$ plane.

Most useful settings are in the range [-2,2], with values less than

-2 corresponding to very little down-weighting, and values greater than

+2 reducing the weighting to natural weighting.

Sidelobe levels and beam-shape degrade with increasing values of

robustness, but the theoretical noise level will also decrease.

The default is no down-weighting (robust=-infinity).

%A line

Standard "line" parameter, with the normal defaults. In particular,

the default is to image all channels. See the help on "line" for

more information.

The "line" parameter consists of a string followed by up to

four numbers, viz:

linetype,nchan,start,width,step

where ``linetype'' is one of "channel", "wide", "velocity" or

"felocity".

%A ref

Line type of the reference channel, specified in a similar to the

"line" parameter. Specifically, it is in the form:

linetype,start,width

Before mapping, the visibility data are divided by the reference

channel. The default is no reference channel.

%A select

This allows a subset of the uv data to be used in the mapping

process. See the Users Manual for information on how to specify

this parameter. The default is to use all data.

%A stokes

Standard polarisation/Stokes parameter selection. See the help

on "stokes" for more information. Several polarisations can be

given. The default is ``ii'' (i.e. Stokes-I, given the

assumption that the source is unpolarised).

%A options

This gives extra processing options. Several options can be

given (abbreviated to uniqueness), and separated by commas:

nocal    Do not apply gains table calibration to the data.

nopol    Do not apply polarisation leakage corrections.

nopass   Do not apply bandpass table calibration to the data.

double   Normally INVERT makes the beam patterns the same

size as the output image. This option causes the

beam patterns to be twice as large.

systemp  Weight each visibility in inverse proportion to the

noise variance. Normally visibilities are weighted in

proportion to integration time. Weighting based on the

noise variance optimises the signal-to-noise ratio

(provided the measures of the system temperature are

reliable!).

mfs      Perform multi-frequency synthesis. The causes all the

channel data to be used in forming a single map. The

frequency dependence of the uv coordinate is thus used to

give better uv coverage and/or avoid frequency

smearing. For this option to produce useful maps, the

intensity change over the frequency band must be small.

You should set the ``line'' parameter to select the

channels that you wish to grid.

sdb      Generate the spectral dirty beam as well as the normal

beam, when MFS processing. The default is to only create

the normal beam. If the spectral dirty beam is created,

this is saved as an extra plane in the beam dataset.

mosaic   Process multiple pointings, and generate a linear

mosaic of these pointings.

imaginary Make imaginary image for non-Hermitian data (holography).

amplitude Produce a image using the data amplitudes only. The

phases of the data are set to zero.

phase     Produce an image using the data phase only. The amplitudes

of the data are set to 1.

%A mode

This determines the algorithm to be used in imaging.

Possible values are:

fft    The conventional grid-and-FFT approach. This is the default

and by far the fastest.

dft    Use a discrete Fourier transform. This avoids aliasing

but at a hugh time penalty.

median This uses a median approach. This is generally robust to

bad data and sidelobes, has a even larger time penalty

and produces images that cannot be deconvolved.

NOTE: Dft and median modes are not supported with options=mosaic.

%A slop

NOTE: This parameter should be used with caution! See the Users

Guide for more information on its applicability.

When forming spectral cubes, INVERT normally insists

that all channels in a given visibility spectrum must be good before

accepting the spectrum for imaging. This keyword allows this rule to

be relaxed. It consists of two parts: a tolerance and a method for

replacing the bad channels.

The tolerance is a value between 0 and 1, giving the fraction of

channels that INVERT will tolerate as being bad before the spectrum

is totally discarded. The default is 0, indicating that INVERT will

not tolerate any bad channels. A value of 1 indicates that INVERT

will accept a spectrum as long as there is at least one good channel.

The replacement method is either the value `zero' or `interpolate',

indicating that the bad channels are either to be replaced with

0, or to be estimated by linear interpolation of two adjacent good

channels. See the Users Guide for the merits and evils of the two

approaches. The default is `zero'.