%N corset

%D Model correlator setups.

%P mchw

%: utility

%B

CORSET is a MIRIAD task to model correlator setups. It takes

as input, an observing frequency and IF and then searches

spectral line files for all spectral lines that are

in IF bandpass. The positions of the lines in spectral windows

and channel numbers and printed and plotted.

%A in

The name of the spectral line file.  Up to 5 input files are

permitted.  Available files are: $MIRCAT/lovas.3mm, $MIRCAT/lovas.1mm

and $MIRCAT/recom.lis. The default is $MIRCAT/lovas.3mm.

%A freq

The rest frequency of the line in GHz. The default 110.0 GHz.

CARMA 10 and 6-m antennas have 85-116 and 215-270 GHz receivers.

3.5-m antennas have SSB 26-36 GHz and 85-115 GHz receivers

%A iffreq

The intermediate frequency in MHz. The default 1500.0 MHz.

This is the frequency in the 1000-9000 MHz IF at which the line

rest frequency will appear. A negative value indictes that

the line should appear in the lower sideband of the first LO.

The IF band is 1-9 GHz at 3 mm and 1-4.5 GHz at 1 mm.

%A vlsr

Vlsr of the source in km/s.  The default 0 km/s.

%A device

The device used for plotting.  See the User's Manual for

details on how to specify the PGPLOT device.

%A log

The output log file.  The default is the terminal.

%A coropt

This keyword identifies the correlator option and is:

0: for crosscorrelation;

1: for autocorrelation;

If this keyword is not present, cross correlation is assumed.

%A corf

Correlator IF frequencies in MHz. Only 4 bands can be plotted at one time.

Default=2000,4000,6000,8000

The IF band is 1-9 GHz at 3 mm and 1-4.5 GHz at 1 mm.

%A corbw

CARMA has 8 bands, yielding a maximum bandwidth of 4GHz per sideband.

Each band may be configured in one of the following bandwidths:

Bandwidth N_chan Chan.width dV[3mm] Vtot[3mm] dV[1mm] Vtot[1mm]

(MHz)   (per sideband)  (MHz)   (km/s)  (km/s)  (km/s)  (km/s)

500     129     3.88    12      1500    3.9     500

250     193     1.30    3.9      750    1.3     250

125     289     0.433   1.3     375     0.43    125

62      385     0.161   0.49    188     0.16    62.5

31      385     0.081   0.24    93.8    0.081   31.2

8       385     0.021   0.061   23.4    0.020   7.81

2       385     0.0052  0.015   5.86    0.005   1.95

An analog filter attenuates the edge channels of the 62-MHz band.

An overlap of about 6 channels is recommended to cover a single line.

The 3.5-meter antennas use a separate correlator with 8 GHz bandwidth,

and 16 500 MHz bands; each of these has 15 channels of 31.25 MHz.

%A birdie

Number of birdies followed by a list of the birdie frequencies

in MHz. Up to 20 birdies are allowed. The default is 0 birdies.