%N linmos

%D Linear mosaicing of datacubes

%P rjs

%: map combination

%B

LINMOS is a MIRIAD task that performs simple linear mosaicing

of input cubes to produce a single output cube.  If only one

input cube is given, LINMOS essentially does primary beam

correction on it.  If the input cubes overlap, LINMOS combines

the overlapping regions so as to minimize the rms error.

To determine the primary beam of the telescope, LINMOS first

checks the map header for the presence of the "pbtype" and then

"pbfwhm" parameters.  If present, LINMOS assumes the primary

beam is the given type.  If these parameters are missing, LINMOS

checks if the telescope is one that it knows.  If so, then the

known form for the primary beam is used. See task "pbplot" to

check LINMOS's primary beam models.

%A in

The names of the input cubes - many may be given.  There is no

default.  Inputs should generally be on the same grid system.

If not, linear interpolation is performed to regrid using the

first image as the template.  LINMOS's ability to do this is

inferior to task REGRID.  The intensity units of all the inputs,

and the pixel size and alignment of the third dimension are

assumed to be the same.

%A out

The name of the output cube.  No default.  The center and pixel

size of the first input image is used as the grid system of the

output.

%A rms

RMS noise level in each of the input cubes.  If not specified,

the value is taken from the 'rms' item in the input image header

if that is available, and if not, then the rms of the previous

image is used.  If no value could be determined for the first

image, a warning is issued and ALL images are given equal weight

by assigning an RMS of 1.0.

%A bw

Bandwidth of the image in GHz, default 0. If specified the beam

response will either be averaged across the frequency band before

being applied to the image or, if the input images contain a

spectral index plane (created with the mfs option of restor)

the images will be evaluated and corrected across the band.

Use this for wide band images to improve the accuracy of the

correction.

Note that doing wide band primary beam correction at low

frequency will make the effective observing frequency vary

significantly across the field.

An optional second parameter can be given to set the number

of frequencies to divide the bandwidth into, it defaults to 10.

%A options

Extra processing options.  Several can be given, separated by

commas.  Minimum match is supported.  Possible values are:

taper        By default, LINMOS fully corrects for primary

beam attenutation.  This can cause excessive

noise amplification at the edge of the mosaiced

field.  The `taper' option aims at achieving

approximately uniform noise across the image.

This prevents full primary beam correction at the

edge of the mosaic.  See Eq. (2) in Sault,

Staveley-Smith and Brouw (1996), A&AS, 120, 375,

or use "options=gains" to see the form of the

tapering.

sensitivity  Rather than a mosaiced image, produce an image

giving the RMS noise across the field.  This is

dependent on the RMS specified, either as an

input parameter or else as a header item (see

above).

gain         Rather than a mosaiced image, produce an image

giving the effective gain across the field.  If

options=taper is used, this will be a smooth

function.  Otherwise it will be 1 or 0 (blanked).

frequency    Rather than a mosaiced image, produce an image

giving the effective frequency across the field.