%N gpnorm

%D Correct the polarisation leakage terms for an offset.

%P rjs

%: calibration

%B

GpNorm is a MIRIAD task which corrects gains and polarisation

leakage tables for XYphase and leakage offsets. It is based on the

supposition that the leakage and gain tables are offset from the

true values. This is often the case when they have been determined

from observations of a weakly polarised calibrator. GpNorm

compares the leakage terms with nominally correct terms, and determines

an leakage and XY phase offset to add which will minimise the

difference between the two sets of leakages. The determined leakage

and XY phase offset can then be used to correct the leakages and

gains.

%A vis

The visibility data-set to correct. This must have a leakage table.

Additionally, it should have a gains table if the XY phase correction

is to be applied. No default.

%A cal

The data-set containing the nominally correct leakage table. No

default.

%A select

Normal uv selection. Only antenna-based selection is supported.

%A options

Task enrichment parameters. Several can be given, separated by commas.

Minimum match is used.

noflag   Treat leakages that are identically 0 as good.

noxy     Do not solve for, or apply, the XY phase offset.

apply    Apply the solved for corrections to the vis data-set. The

default is to solve for them only.

xyleak   GPNORM can determine the xy phase differences from both the

gains table and polarisation leakages. By default, it assumes

the gains-derived value should take precedence. The xyleak

option causes GPNORM to take the leakage-derived value as the

preferred on. This only has an effect if options=apply is used

and options=noxy is not used.