%N gpcal

%D Gain/phase/polarization calibration of dual feed data.

%P rjs

%: calibration

%B

Gpcal is a MIRIAD task which determines calibration corrections

(both antenna gains and instrumental polarisation characteristics)

for an array with dual feeds, from an observation of a point

source. The source can be polarised, with unknown polarisation

characteristics. Though the source may be strongly polarized, the

instrumental polarisation errors are assumed to be small (of order

at most a few percent).

Normally GPCAL writes the solutions as a gains table (item `gains')

and a polarization leakage table (item `leakage').

GPCAL can handle either dual linear or dual circular feeds. However

by default it expects dual linears -- you must use options=circular

so switch it to circular mode. Also the terminology in this document

is aimed at linears (e.g. we talk of ``XY phase'' -- for dual circulars

this is really ``RL phase'').

Note that the user specifies which parameters are to be solved

for. In the case of leakages and xyphases, GPCAL will check for

the existence of items ``leakage'' and ``gains'' in the input

data-set. If present, these will be used as the initial estimates

of these parameters. If you are not solving for these parameters,

they will be held at there initial value through the solution

process. After converging on a solution, and if the xyphase offsets

or leakage parameters have been modified, GPCAL will write out

their current values to the appropriate items.

Conventions: Unfortunately there has been a number of changes in

the `sign conventions' used within Miriad. For a discussion of the

conventions, past sign errors and how they affect you, see the

memo ``The Sign of Stokes-V, etc'' by Bob Sault.

%A vis

Input visibility data file. The data should be either raw linear

or raw circular polarisations. No default. The visibility data

must be in time order.

%A select

Standard uv selection. Default is all data.

%A line

Standard line-type specification. Multiple channels can be given.

Generally it is better to give multiple channels, rather than

averaging them into a `channel-0'. The default is all the channel

data (or all the wide data, if there is no channel data).

%A flux

The values of the I,Q,U,V Stokes parameters. If no values are

given, and it is a source known to GPCAL, GPCAL uses its known

flux as the default. If GPCAL does not know the source, the

flux is determined by assuming that the rms gain amplitude is 1.

If the option `qusolve' is used, the given fluxes for Q and U are

used as the initial estimates. Also see the "oldflux" option.

%A refant

The reference antenna. Default is 3. The reference antenna needs to be

present throughout the observation. Any solution intervals where the

reference antenna is missing are discarded.

%A minants

The minimum number of antenna that must be present before a

solution is attempted. Default is 2.

%A interval

This gives one or two numbers, both given in minutes, both being used

to determine the extents of an amplitude calibration solution interval.

The first gives the max length of a solution interval. The second

gives the max gap size in a solution interval. A new solution interval

is started when either the max time length is exceeded, or a gap

larger than the max gap is encountered. The default is max length

is 5 minutes, and the max gap size is the same as the max length.

The polarisation characteristics are assumed to be

constant over the observation.

%A tol

Error tolerance. The default is 0.001, which should be adequate.

%A xyphase

Generally the use of this parameter has been superceded.

Initial estimate of the XY phase of each antenna. The default is

0 for all antennas. If the XY phase has not been applied to the

data, then it is important that this parameter is set correctly,

particularly for the reference antenna.

%A options

These options determine what GPCAL solves for. There are many

permutations, the more obscure or useless of which are not

supported. The option values are used to turn on or off some

of the solvers. Several options can be given, separated by

commas. Minimum match is used. Some combinations of these options

are not supported.

xyvary     The XY phase varies with time. By default the XY phase

is assumed to remain constant.

qusolve    Solve for Q and U fluxes. Good parallactic

angle coverage is required for this.

oldflux    This causes GPCAL to use a pre-August 1994 ATCA flux

density scale. See the help on "oldflux" for more

information.

circular   Expect/handle circularly polarised data.

nopol      Do not solve for the instrumental polarisation

leakage characteristics. The default is to solve

for the polarisation leakages on all feeds except

the X feed of the reference antenna.

noxy       Do not solve for any XY phase offset. The default is to

solve for the XY phase offset on all antennas

except for the reference antenna.

nopass     Do not apply bandpass correction. The default is

to apply bandpass correction if possible. This is

rarely useful.

noamphase  Do not solve for the amplitude and phase. The

default is to solve for amplitude and phase. This is

rarely useful.

The following are options for advanced users, and should be used

with caution. Contact Bob Sault for guidance on their applicability.

xyref      Solve for the XY phase of the reference antenna. To

do this, the source must be linearly polarized and you

must have reasonable parallactic angle coverage. This

option can be used with ``noxy'', in which case GPCAL

solves for the offset of the reference antenna.

polref     Solve for the instrumental polarization leakage

of the X feed on the reference antenna. This can

be combined with ``nopol'', in which case GPCAL

solves for X feed of the reference antenna only.

vsolve     Solve for the Stokes-V of the source. This is only

possible for linear feeds and a preliminary leakage

solution for the observation already exists. This

preliminary solution must be formed from a

calibrator with known Stokes-V.